# Cyber Security – Electronic Security Perimeter(s)

# CIP-005-AB-7

# Standard Effective Date: October 1, 2024

## Audit Summary

| Entity | Entity name; or the ISO. |
| --- | --- |
| Functional Entity Type(s) | Functional entity types applicable to the Entity during the audit period; or the ISO. |
| Audit ID |  |
| Audit Period (Audit start date or standard effective date, whichever comes later) (Audit end date or standard withdrawal/supersede date, whichever comes first) | From: Click or tap to enter a date. To: Click or tap to enter a date. |
| Audit Assessment Start Date (Scheduled or Spot Check) | Click or tap to enter a date. |
| Compliance Monitoring Entity | Alberta Electric System Operator (AESO) for Alberta Entities or WECC on behalf of the Market Surveillance Administrator (MSA) for the ISO. |
| Date of Audit Completion | Click or tap to enter a date. |

The Reliability Standard Audit Worksheet (RSAW) is not an authoritative document. It adds transparency and encourages consistency in the Alberta Reliability Standard monitoring processes. RSAWs provide information related to the compliance assessment approach for each Alberta Reliability Standard. The Compliance Monitoring Entity may apply additional monitoring and assessment methods not documented in the RSAW and reserves the right to request additional evidence not listed in the RSAW. The Entity completes the applicable fields and submits the RSAW to the Compliance Monitoring Entity as requested during a compliance monitoring activity. The Compliance Monitoring Entity completes the document and its compliance assessment in the auditor notes section and retains it as an audit working paper to support the audit conclusions.

### Applicability of Requirements1

|  | Req. Removed | ISO | LO AGF | OP AGF | LO GU | OP GU | LO TF | OP TF | LO DS | OP DS | MP DTS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 |  | X | X2 | X3 | X4 | X5 | X | X | X6 |  |  |
| R2 |  | X | X2 | X3 | X4 | X5 | X | X | X6 |  |  |
| R3 |  | X | X2 | X3 | X4 | X5 | X | X | X6 |  |  |

LO AGF - Legal owner of an aggregated generating facility

OP AGF - Operator of an aggregated generating facility

LO GU - Legal owner of a generating unit

OP GU - Operator of a generating unit

LO TF - Legal owner of a transmission facility

OP TF - Operator of a transmission facility

LO DS - Legal owner of an electric distribution system

OP DS - Operator of an electric distribution system

MP DTS - Market participant receiving service under Rate DTS of the ISO tariff

### Applicability of Requirements Qualifiers, if applicable: 1 For the purpose of the requirements contained herein, the list of functional entities marked in the table above will be collectively referred to as “Responsible Entities.” For requirements in this reliability standard where a specific functional entity or subset of functional entities are the applicable entity or entities, the functional entity or entities are specified explicitly. Additionally, for the purpose of the requirements contained herein, the following facilities, systems, and equipment owned by each Responsible Entity marked in the table above are those to which these requirements are applicable. For requirements in this reliability standard where a specific type of facilities, system, or equipment or subset of facilities, systems, and equipment are applicable, these are specified explicitly. - Legal owner of an electric distribution system and legal owner of a transmission facility: One or more of the following facilities, systems and equipment that operate at, or control elements that operate at, a nominal voltage of 25 kV or less and are owned by a legal owner of an electric distribution system or a legal owner of a transmission facility for the protection or restoration of the bulk electric system: - Each underfrequency load shedding or under voltage load shed system that: - Is part of a load shedding program that is subject to one or more requirements in a reliability standard; and - Performs automatic load shedding under a common control system owned by the Responsible Entity, without human operator initiation, of 300 MW or more. - Each remedial action scheme where the remedial action scheme is subject to one or more requirements in a reliability standard. - Each protection system (excluding underfrequency load shedding and under voltage load shed) that applies to any transmission facility or electric distribution system where the protection system is subject to one or more requirements in a reliability standard. - Each cranking path and group of elements meeting the initial switching requirements from a blackstart resource up to and including the first point of connection of the starting station service of the next generating unit(s) or aggregated generating facility(ies) to be started. - Responsible Entities marked in the table above other than a legal owner of an electric distribution system: all bulk electric system facilities - Exemptions: The following are exempt from CIP-005-AB-7: - Cyber assets at facilities regulated by the Canadian Nuclear Safety Commission. - Cyber assets associated with communication networks and data communication links between discrete electronic security perimeters. - [Intentionally left blank.] - For the legal owner of an electric distribution system, the systems and equipment that are not included in section (a) above. - Responsible Entities that identify that they have no BES cyber systems categorized as high impact or medium impact according to the CIP-002 identification and categorization processes. 2 The legal owner of an aggregated generating facility that is part of the bulk electric system. 3 The operator of an aggregated generating facility that is part of the bulk electric system. 4 The legal owner of a generating unit that is part of the bulk electric system. 5 The operator of a generating unit that is part of the bulk electric system. 6 A legal owner of an electric distribution system that owns one or more of the following facilities, systems, and equipment for the protection or restoration of the bulk electric system: - Each underfrequency load shedding or under voltage load shed system that: - Is part of a load shedding program that is subject to one or more requirements in a reliability standard; and - Performs automatic load shedding under a common control system owned by the Responsible Entity, without human operator initiation, of 300 MW or more. - Each remedial action scheme where the remedial action scheme is subject to one or more requirements in a reliability standard; - Each protection system (excluding underfrequency load shedding and under voltage load shed) that applies to an electric distribution system where the protection system is subject to one or more requirements in a reliability standard; and - Each cranking path and group of elements meeting the initial switching requirements from a blackstart resource up to and including the first point of connection of the starting station service of the next generating unit(s) or aggregated generating facility(ies) to be started.

### Findings

(This section to be completed by the Compliance Monitoring Entity)

| Req. | Finding | Summary and Documentation | Functions Monitored |
| --- | --- | --- | --- |
| R1 |  |  |  |
| Part 1.1 |  |  |  |
| Part 1.2 |  |  |  |
| Part 1.3 |  |  |  |
| Part 1.4 |  |  |  |
| Part 1.5 |  |  |  |
| R2 |  |  |  |
| Part 2.1 |  |  |  |
| Part 2.2 |  |  |  |
| Part 2.3 |  |  |  |
| Part 2.4 |  |  |  |
| Part 2.5 |  |  |  |
| R3 |  |  |  |
| Part 3.1 |  |  |  |
| Part 3.2 |  |  |  |

| Req. | Areas of Concern |
| --- | --- |

| Req. | Recommendations |
| --- | --- |

| Req. | (Positive) Observations |
| --- | --- |

### Subject Matter Experts

Identify the Subject Matter Expert(s) responsible for this Reliability Standard.

Entity Response (Required; Insert additional rows if needed):

| SME Name | Title | Organization | Requirement(s) |
| --- | --- | --- | --- |

## R1 Supporting Evidence and Documentation

R1.	Each Responsible Entity shall implement one or more documented processes that collectively include each of the applicable requirement parts in CIP-005-AB-7 Table R1 – Electronic Security Perimeter. [Alberta Risk Rating: Medium] [Time Horizon: Operations Planning and Same Day Operations].

M1.	Evidence must include each of the applicable documented processes that collectively include each of the applicable requirement parts in CIP-005-AB-7 Table R1 – Electronic Security Perimeter and additional evidence to demonstrate implementation as described in the Measures column of the table.

## R1 Part 1.1

| CIP-005-AB-7 Table R1 – Electronic Security Perimeter |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 1.1 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems and their associated: • protected cyber assets | All applicable cyber assets connected to a network via a routable protocol shall reside within a defined electronic security perimeter. | An example of evidence may include, but is not limited to, a list of all electronic security perimeters with all uniquely identifiable applicable cyber assets connected via a routable protocol within each electronic security perimeter. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R1, Part 1.1

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 1.1) Verify the Responsible Entity has documented one or more process(es) which require all applicable cyber assets connected to a network via a routable protocol to reside within a defined electronic security perimeter. |
| --- | --- |
|  | (Part 1.1) Verify each applicable cyber asset connected to a network via a routable protocol resides within a defined electronic security perimeter. |
|  | (Part 1.1) For each defined electronic security perimeter, verify the identification of any associated protected cyber assets. |
| Note to Auditor: - This Part is applicable to all high and medium impact BES cyber systems and their associated protected cyber assets regardless of external routable connectivity. - Those cyber assets that are part of a high or medium impact BES cyber system that are not connected to a network via a routable protocol need not reside within a defined electronic security perimeter. - For cyber assets that are part of a high or medium impact BES cyber system that do not reside within a defined electronic security perimeter, the absence of a connection to a network via a routable protocol will be verified. - The reason to identify an electronic security perimeter without external routable connectivity is to identify the protected cyber assets associated with the electronic security perimeter. - In order to verify that each cyber asset residing within a defined electronic security perimeter has been identified as either a BES cyber asset or as a protected cyber asset, it may be necessary to examine the electronic security perimeter and conduct an inventory of network connections within the electronic security perimeter. - The impact rating of protected cyber assets is equal to the highest rated BES cyber system in the same defined electronic security perimeter. |  |

### Auditor Notes:

## R1 Part 1.2

| CIP-005-AB-7 Table R1 – Electronic Security Perimeter |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 1.2 | High Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | All external routable connectivity must be through an identified electronic access point. | An example of evidence may include, but is not limited to, network diagrams showing all external routable communication paths and the identified electronic access points. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R1, Part 1.2

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 1.2) Verify the Responsible Entity has documented one or more processes which ensure all external routable connectivity is through an identified electronic access point. |
| --- | --- |
|  | (Part 1.2) Verify that all external routable connectivity is through an identified electronic access point. |
|  | (Part 1.2) For each defined electronic security perimeter without an identified electronic access point, verify that no external routable connectivity exists. |
| Note to Auditor: |  |

### Auditor Notes:

## R1 Part 1.3

| CIP-005-AB-7 Table R1 – Electronic Security Perimeter |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 1.3 | Electronic access points for High Impact BES cyber systems Electronic access points for Medium Impact BES cyber systems | Require inbound and outbound access permissions, including the reason for granting access, and deny all other access by default. | An example of evidence may include, but is not limited to, a list of rules (firewall, access control lists, etc.) that demonstrate that only permitted access is allowed and that each access rule has a documented reason. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R1, Part 1.3

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 1.3) Verify the Responsible Entity has documented one or more processes which require inbound and outbound access permissions, including the reason for granting access, and deny all other access by default. |
| --- | --- |
|  | (Part 1.3) For each Applicable System, verify inbound and outbound access permissions are implemented. |
|  | (Part 1.3) For each Applicable System, verify each inbound and each outbound access permission includes the reason for granting access. |
|  | (Part 1.3) For each Applicable System, verify inbound and outbound access is denied by default. |
| Note to Auditor: Some vendor firewalls contain an implicit “deny” statement in the Access Control List (ACL). Vendor documentation of that configuration is a form of evidence to demonstrate the deny-by-default requirement within this Part. |  |

### Auditor Notes:

## R1 Part 1.4

| CIP-005-AB-7 Table R1 – Electronic Security Perimeter |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 1.4 | High Impact BES cyber systems with dial-up connectivity and their associated: • protected cyber assets Medium Impact BES cyber systems with dial-up connectivity and their associated: • protected cyber assets | Where technically feasible, perform authentication when establishing dial-up connectivity with applicable cyber assets. | An example of evidence may include, but is not limited to, a documented process that describes how the Responsible Entity is providing authenticated access through each dial-up connection. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R1, Part 1.4

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 1.4) Verify the Responsible Entity has documented one or more processes to perform authentication when establishing dial-up connectivity with applicable cyber assets, where technically feasible. |
| --- | --- |
|  | (Part 1.4) For each cyber asset of an Applicable System, verify authentication is performed when establishing dial-up connectivity, or that an approved technical feasibility exception covers the cyber asset. |
|  | (Part 1.4) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: If the Responsible Entity does not have or does not allow dial-up connectivity, the Responsible Entity is not required to document one or more processes to perform authentication when establishing dial-up connectivity with applicable cyber assets. It is sufficient to verify that the Responsible Entity does not have dial-up connectivity. |  |

### Auditor Notes:

## R1 Part 1.5

| CIP-005-AB-7 Table R1 – Electronic Security Perimeter |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 1.5 | Electronic access points for High Impact BES cyber systems Electronic access points for Medium Impact BES cyber systems at control centres | Have one or more methods for detecting known or suspected malicious communications for both inbound and outbound communications. | An example of evidence may include, but is not limited to, documentation that malicious communications detection methods (e.g. intrusion detection system, application layer firewall, etc.) are implemented. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R1, Part 1.5

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 1.5) Verify the Responsible Entity has documented one or more processes which include one or more methods for detecting known or suspected malicious communications for both inbound and outbound communications. |
| --- | --- |
|  | (Part 1.5) For each Applicable System, verify the Responsible Entity has implemented one or more methods for detecting known or suspected malicious communications for both inbound and outbound communications. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Supporting Evidence and Documentation

R2.	Each Responsible Entity shall implement one or more documented processes that collectively include the applicable requirement parts, where technically feasible, in CIP-005-AB-7 Table R2 –Remote Access Management. [Alberta Risk Rating: Medium] [Time Horizon: Operations Planning and Same Day Operations].

M2.	Evidence must include the documented processes that collectively address each of the applicable requirement parts in CIP-005-AB-7 Table R2 –Remote Access Management and additional evidence to demonstrate implementation as described in the Measures column of the table.

## R2 Part 2.1

| CIP-005-AB-7 Table R2 – Remote Access Management |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 2.1 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | For all interactive remote access, utilize an intermediate system such that the cyber asset initiating interactive remote access does not directly access an applicable cyber asset. | Examples of evidence may include, but are not limited to, network diagrams or architecture documents. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R2, Part 2.1

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 2.1) Verify the Responsible Entity has documented one or more processes which require the utilization of an intermediate system such that the cyber asset initiating interactive remote access does not directly access an applicable cyber asset. |
| --- | --- |
|  | (Part 2.1) Verify all interactive remote access utilizes an intermediate system, or that an approved technical feasibility exception covers this circumstance. |
|  | (Part 2.1) Verify that the cyber asset initiating interactive remote access does not directly access an applicable cyber asset, or that an approved technical feasibility exception covers this circumstance. |
|  | (Part 2.1) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Part 2.2

| CIP-005-AB-7 Table R2 – Remote Access Management |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 2.2 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | For all interactive remote access sessions, utilize encryption that terminates at an intermediate system. | An example of evidence may include, but is not limited to, architecture documents detailing where encryption initiates and terminates. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R2, Part 2.2

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 2.2) Verify the Responsible Entity has documented one or more processes which utilize encryption that terminates at an intermediate system for all interactive remote access sessions. |
| --- | --- |
|  | (Part 2.2) Verify all interactive remote access utilizes encryption that terminates at an intermediate system, or that an approved technical feasibility exception covers this circumstance. |
|  | (Part 2.2) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Part 2.3

| CIP-005-AB-7 Table R2 – Remote Access Management |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 2.3 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | Require multi-factor authentication for all interactive remote access sessions. | An example of evidence may include, but is not limited to, architecture documents detailing the authentication factors used. Examples of authenticators may include, but are not limited to, • Something the individual knows such as passwords or PINs. This does not include User ID; • Something the individual has such as tokens, digital certificates, or smart cards; or • Something the individual is such as fingerprints, iris scans, or other biometric characteristics. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R2, Part 2.3

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 2.3) Verify the Responsible Entity has documented one or more processes which require multi-factor authentication for all interactive remote access sessions. |
| --- | --- |
|  | (Part 2.3) Verify all interactive remote access sessions require multi-factor authentication, or that an approved technical feasibility exception covers this circumstance. |
|  | (Part 2.3) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Part 2.4

| CIP-005-AB-7 Table R2 – Remote Access Management |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 2.4 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | Have one or more methods for determining active vendor remote access sessions (including interactive remote access and system-to-system remote access). | Examples of evidence may include, but are not limited to, documentation of the methods used to determine active vendor remote access (including interactive remote access and system-to-system remote access), such as: • Methods for accessing logged or monitoring information to determine active vendor remote access sessions; • Methods for monitoring activity (e.g. connection tables or rule hit counters in a firewall, or user activity monitoring) or open ports (e.g. netstat or related commands to display currently active ports) to determine active system to system remote access sessions; or • Methods that control vendor initiation of remote access such as vendors calling and requesting a second factor in order to initiate remote access. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R2, Part 2.4

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 2.4) Verify the Responsible Entity has documented one or more processes which have one or more methods for determining active vendor remote access sessions (including interactive remote access and system-to-system remote access). |
| --- | --- |
|  | (Part 2.4) Verify all active vendor remote access sessions (including interactive remote access and system-to-system remote access) can be determined, or that an approved technical feasibility exception covers the active vendor remote access sessions that cannot be determined. |
|  | (Part 2.4) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: |  |

### Auditor Notes:

## R2 Part 2.5

| CIP-005-AB-7 Table R2 – Remote Access Management |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 2.5 | High Impact BES cyber systems and their associated: • protected cyber assets Medium Impact BES cyber systems with external routable connectivity and their associated: • protected cyber assets | Have one or more method(s) to disable active vendor remote access (including interactive remote access and system-to-system remote access). | Examples of evidence may include, but are not limited to, documentation of the methods(s) used to disable active vendor remote access (including interactive remote access and system-to-system remote access), such as: • Methods to disable vendor remote access at the applicable electronic access point for system-to-system remote access; or • Methods to disable vendor interactive remote access at the applicable intermediate system. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R2, Part 2.5

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 2.5) Verify the Responsible Entity has documented one or more processes which have one or more method(s) to disable active vendor remote access (including interactive remote access and system-to-system remote access). |
| --- | --- |
|  | (Part 2.5) Verify the Responsible Entity has one or more methods to disable active vendor remote access (including interactive remote access and system-to-system remote access). |
|  | (Part 2.5) If a technical feasibility exception is applicable to this Part, verify the compensating measures identified by the technical feasibility exception are implemented. |
| Note to Auditor: |  |

### Auditor Notes:

## R3 Supporting Evidence and Documentation

R3.	Each Responsible Entity shall implement one or more documented processes that collectively include the applicable requirement parts in CIP-005-AB-7 Table R3 –Vendor Remote Access Management for Electronic Access Control or Monitoring Systems and Physical Access Control Systems. [Alberta Risk Rating: Medium] [Time Horizon: Operations Planning and Same Day Operations].

M3.	Evidence must include the documented processes that collectively address each of the applicable requirement parts in CIP-005-AB-7 Table R3 – Vendor Remote Access Management for Electronic Access Control or Monitoring Systems and Physical Access Control Systems and additional evidence to demonstrate implementation as described in the Measures column of the table.

## R3 Part 3.1

| CIP-005-AB-7 Table R3 – Vendor Remote Access Management for Electronic Access Control or Monitoring Systems and Physical Access Control Systems |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 3.1 | Electronic access control or monitoring systems and physical access control systems associated with High Impact BES cyber systems Electronic access control or monitoring systems and physical access control systems associated with Medium Impact BES cyber systems with external routable connectivity | Have one or more method(s) to determine authenticated vendor- initiated remote connections. | Examples of evidence may include, but are not limited to, documentation of the methods used to determine authenticated vendor-initiated remote connections, such as: • Methods for accessing logged or monitoring information to determine authenticated vendor-initiated remote connections. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R3, Part 3.1

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 3.1) Verify the Responsible Entity has documented one or more processes which have one or more method(s) to determine authenticated vendor- initiated remote connections. |
| --- | --- |
|  | (Part 3.1) Verify the Responsible Entity is capable of determining authenticated vendor-initiated remote connections by implementing the documented process either: - Continuously, providing connection information in real time. In this case evidence of the continuous monitoring should be reviewed to verify the capability. - On demand, initiated as needed by the Responsible Entity. In this case, a live demonstration or other means of verification of the ability may be used. |
| Note to Auditor: |  |

### Auditor Notes:

## R3 Part 3.2

| CIP-005-AB-7 Table R3 – Vendor Remote Access Management for Electronic Access Control or Monitoring Systems and Physical Access Control Systems |  |  |  |
| --- | --- | --- | --- |
| Part | Applicable Systems | Requirements | Measures |
| 3.2 | Electronic access control or monitoring systems and physical access control systems associated with High Impact BES cyber systems Electronic access control or monitoring systems and physical access control systems associated with Medium Impact BES cyber systems with external routable connectivity | Have one or more method(s) to terminate authenticated vendor- initiated remote connections and control the ability to reconnect. | Examples of evidence may include, but are not limited to, documentation of the methods(s) used to terminate authenticated vendor-initiated remote connections to applicable systems. Examples include terminating an active vendor-initiated shell/process/session or dropping an active vendor-initiated connection in a firewall. Methods to control the ability to reconnect, if necessary, could be: disabling an Active Directory account; disabling a security token; restricting IP addresses from vendor sources in a firewall; or physically disconnecting a network cable to prevent a reconnection. |

### Entity Response (Required):

Compliance Narrative:

Provide a brief explanation of how you comply with this Requirement. References to supplied evidence, including links to the appropriate page are recommended.

### Entity Evidence (Required):

| The following information is requested for each document submitted as evidence. Also, evidence submitted should be highlighted and bookmarked/annotated, as appropriate, to identify the exact location where evidence of compliance may be found. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| File Name | Document Title | Revision or Version | Document Date | Relevant Page(s) or Section(s) | Description of Applicability of Document |

### Audit Team Evidence Reviewed

(This section to be completed by the Compliance Monitoring Entity):

### Compliance Assessment Approach Specific to CIP-005-AB-7, R3, Part 3.2

(This section to be completed by the Compliance Monitoring Entity):

|  | (Part 3.2) Verify the Responsible Entity has documented one or more processes which have one or more method(s) to terminate authenticated vendor-initiated remote connections. |
| --- | --- |
|  | (Part 3.2) Verify the Responsible Entity has documented one or more processes which have one or more method(s) to control the ability to reconnect authenticated vendor-initiated remote connections. |
|  | (Part 3.2) Verify the Responsible Entity is capable of terminating authenticated vendor-initiated remote connections by implementing the documented process. |
|  | (Part 3.2) Verify the Responsible Entity is capable of controlling the ability to reconnect authenticated vendor-initiated remote connections by implementing the documented process. |
| Note to Auditor: |  |

### Auditor Notes:

# Additional Information

Bolded terms in the Alberta Reliability Standard RSAW refer to terms in the Consolidated Authoritative Document Glossary.

# Revision History for RSAW

| Version | Issue Date | Description |
| --- | --- | --- |
| 1.0 | 2026-04-01 | Initial version |